SMART INDIA HACKATHON 2026

Problem Statement ID: 26085

Urban Flood Nowcasting System

Drainage and Rainfall Coupling

─────────────────────────────────────

Comprehensive Project Proposal & Technical Architecture

Ministry of Earth Sciences (MoES)  |  NCMRWF

Theme: Disaster Management  |  Category: Software

Prototype City: Chennai, Tamil Nadu

# 1. Executive Summary

Urban flooding in Indian cities is no longer a rare disaster — it is an annual crisis. Cities like Mumbai, Delhi, Chennai, and Hyderabad suffer severe flooding every monsoon season, causing loss of life, economic disruption, and infrastructure damage. Despite advances in weather forecasting, municipal bodies still lack the ability to predict exactly which streets will flood, and when.

This project proposes the development of a real-time Urban Flood Nowcasting System — an AI and physics-based platform that predicts street-level flooding 0 to 3 hours in advance. The system fuses real-time rainfall nowcasts from Doppler weather radar with high-resolution Digital Elevation Models (DEM) and a graph-based mathematical model of the city's underground drainage network.

The prototype is designed for the city of Chennai, leveraging the well-documented 2015 flood event as the primary training and validation dataset. The system delivers street-level water depth predictions, a live GIS dashboard for municipal authorities, automated alerts, and a Route Safety API that navigation applications can use to suggest flood-free routes to commuters and emergency services.

# 2. Problem Statement Analysis

## 2.1 The Core Problem

Traditional weather forecasting systems answer one question: how much rain will fall? They do not answer the question that actually matters for urban disaster response: where will the streets flood?

Urban flooding is a hyper-local phenomenon determined by three interacting factors that standard weather models ignore entirely:

▸  Micro-topography — the precise elevation profile of streets, intersections, and underpasses determines where water accumulates.

▸  Surface imperviousness — concrete roads, asphalt, and paved surfaces absorb almost no rainfall, converting 70–95% of rainfall directly into surface runoff.

▸  Drainage network capacity — underground pipes, manholes, and canals have fixed hydraulic capacities. When inflow exceeds capacity, water backs up and overflows onto streets.

The absence of a system that couples these three factors means municipalities are always reacting to flooding rather than preventing it. Resources cannot be pre-positioned, evacuation routes are not identified in advance, and emergency vehicles get trapped in flooded areas.

## 2.2 Who is Affected

| Stakeholder | Current Pain | What They Need |
| --- | --- | --- |
| Municipal Corporations | No advance warning of which areas to protect | 0-3 hour ahead flood location predictions |
| Traffic Police | Roads blocked unexpectedly, no alternate routing | Real-time flood-safe route guidance |
| Emergency Services | Ambulances and fire engines get stuck in floods | API for flood-aware navigation |
| Commuters & Citizens | No warning before roads submerge | Mobile alerts for their area |
| Disaster Management Authorities | Reactive deployment of resources | Predictive dashboard to pre-deploy teams |
| Urban Planners | No data on which drains fail first | Historical pattern data for infrastructure planning |

## 2.3 Why Chennai

▸  The 2015 Chennai floods are the most comprehensively documented urban flood event in Indian history — ground truth data, satellite imagery, and news records are all available.

▸  Chennai's flooding mechanism is exactly what this PS targets: heavy rainfall overwhelming the drainage network, not lake breaches or river flooding.

▸  Flat terrain makes DEM-based surface flow modelling more accurate and computationally straightforward.

▸  IMD Chennai Doppler radar provides good historical radar data for the study area.

▸  Sentinel-1 SAR satellite imagery clearly shows the November-December 2015 inundation extent — ideal for model validation.

▸  Multiple academic research papers on Chennai 2015 floods are available as reference for hydraulic parameters.

# 3. Proposed Solution Overview

The proposed solution is a coupled, real-time flood prediction platform built on three pillars:

▸  Pillar 1 — Physics-Based Modelling: Hydrological and hydraulic equations govern how water moves on surfaces and through pipes. This ensures predictions are grounded in physical reality, not statistical approximation.

▸  Pillar 2 — Machine Learning: ML models learn from historical flood events to predict flood depth and risk classification at each street segment, improving accuracy over time.

▸  Pillar 3 — Real-Time Data Fusion: Radar rainfall, live sensor data, and terrain information are continuously fused to update predictions every 5 to 15 minutes.

The system is unique in that it goes beyond standard models by incorporating three additional factors that materially affect flood risk in Indian cities: road surface permeability, drainage infrastructure condition and age, and tree canopy interception of rainfall. These factors are rarely included in existing systems but significantly improve street-level prediction accuracy.

# 4. End-to-End System Architecture

## 4.1 Architecture Overview

The system is organized into five layers, each with a distinct responsibility. Data flows from left to right through these layers in a continuous pipeline:

| Layer | Name | Responsibility |
| --- | --- | --- |
| Layer 1 | Data Ingestion | Collect and preprocess all raw data inputs |
| Layer 2 | Feature Engineering | Transform raw data into model-ready features |
| Layer 3 | Prediction Engine | Run physics and ML models to generate predictions |
| Layer 4 | Output Services | Serve predictions via dashboard, alerts, and API |
| Layer 5 | Feedback Loop | Capture actual outcomes and retrain models |

## 4.2 Layer 1 — Data Ingestion

This layer is responsible for pulling all input data from external sources, cleaning it, and storing it in standardized formats for downstream processing.

### 4.2.1 Rainfall Data Ingestion

▸  Source: IMD Doppler Weather Radar, MOSDAC satellite data, IMD rain gauge network

▸  Format: NetCDF and HDF5 radar grid files, CSV gauge readings

▸  Frequency: Radar data refreshes every 5 to 15 minutes

▸  Processing: Radar quality control (remove beam blockage, ground clutter), spatial interpolation from gauge points, unit conversion to mm/hr per grid cell

▸  Storage: PostGIS raster tables with timestamp indexing

### 4.2.2 Terrain Data Ingestion

▸  Source: Cartosat-2 DEM from ISRO Bhuvan portal (primary), SRTM 30m DEM from USGS EarthExplorer (fallback)

▸  Format: GeoTIFF raster files

▸  Processing: Reprojection to UTM Zone 44N, void filling, depression filling using hydrological conditioning, flow direction and flow accumulation raster generation using D8 algorithm

▸  Output: Flow direction grid, flow accumulation grid, slope grid, pit-filled DEM

### 4.2.3 Drainage Network Ingestion

▸  Source: OpenStreetMap drainage layer, CMWSSB (Chennai Metropolitan Water Supply and Sewerage Board) network data where available

▸  Format: Shapefile / GeoJSON for vector network data

▸  Processing: Network topology cleaning, direction assignment based on elevation, capacity estimation from pipe diameter using Manning's equation, node and edge attribute assignment

▸  Storage: PostGIS network tables with NetworkX graph serialization

### 4.2.4 Land Surface and Road Data Ingestion

▸  Source: ESA Copernicus LULC map, OpenStreetMap road network with surface tags, Sentinel-2 satellite imagery for NDVI computation

▸  Processing: LULC reclassification to runoff coefficient values, road surface type mapping to permeability values, NDVI to canopy interception factor conversion

▸  Frequency: Static layers updated monthly, NDVI updated weekly

## 4.3 Layer 2 — Feature Engineering

Raw data from Layer 1 is transformed into the specific numerical features required by physics models and ML models. This layer runs every time new radar data arrives.

### 4.3.1 Runoff Coefficient Computation

Each grid cell in the study area is assigned a composite runoff coefficient based on three sub-factors:

| Factor | Data Source | Impact on Runoff |
| --- | --- | --- |
| Road surface material | OSM surface tags + LULC | Concrete/asphalt = 0.85-0.95, Paver = 0.50-0.70, Gravel = 0.30-0.55 |
| Tree canopy cover | Sentinel-2 NDVI | High canopy = 15-40% interception reduction in effective rainfall |
| Soil/land type | LULC map | Built-up = high runoff, Green space = low runoff, Water body = zero infiltration |

The composite formula used is the Rational Method:

Q = C × I × A

Where Q is peak runoff in cubic metres per second, C is the composite runoff coefficient (0.0 to 1.0), I is rainfall intensity in mm/hr, and A is the contributing area in hectares.

### 4.3.2 Drain Condition Factor

Each drain pipe segment is assigned an effective capacity that accounts for physical degradation:

▸  Design capacity is calculated from pipe diameter and slope using Manning's equation

▸  Condition factor is applied based on pipe age and material: new concrete pipe = 0.95, 20-year brick masonry = 0.65, 40-year earthen drain = 0.40

▸  Blockage probability based on historical maintenance records and street-level garbage density is applied as an additional derating factor

▸  Effective capacity = Design capacity × Condition factor × (1 - Blockage probability)

### 4.3.3 Graph Feature Vectors

For each node and edge in the drain graph, a feature vector is assembled:

▸  Node features: elevation, upstream contributing area, number of connected pipes, historical overflow frequency, proximity to large open drains or nalas

▸  Edge features: pipe length, diameter, material, slope, effective capacity, current flow utilization ratio, upstream load, condition factor

## 4.4 Layer 3 — Prediction Engine

This is the core computational layer. It contains four models that run in sequence, each feeding results into the next.

### 4.4.1 Model 1 — Rainfall Nowcasting (ConvLSTM)

| Attribute | Detail |
| --- | --- |
| Purpose | Predict rainfall intensity for next 0 to 3 hours from current radar data |
| Algorithm | ConvLSTM (Convolutional Long Short-Term Memory Neural Network) |
| Why this algorithm | Radar data is both spatial (a 2D grid) and temporal (a sequence of frames). ConvLSTM processes both dimensions simultaneously, capturing how rainfall cells move and intensify across space and time |
| Input | Sequence of 12 radar rainfall grid frames from past 3 hours (one frame every 15 minutes), each frame is a 128×128 grid of mm/hr values |
| Output | 12 predicted rainfall frames for next 3 hours at 15-minute intervals |
| Training data | 5 years of IMD Chennai radar data including major events of 2015, 2017, 2021 |
| Alternative if complex | Skip building this model and consume IMD's own nowcast output directly as input to the pipeline. This is a valid prototype shortcut. |

### 4.4.2 Model 2 — Surface Runoff Routing (Physics-Based)

| Attribute | Detail |
| --- | --- |
| Purpose | Route rainfall across the terrain surface to calculate how much water reaches each drain inlet |
| Algorithm | Rational Method combined with D8 flow routing algorithm on DEM |
| Why physics not ML | Surface flow follows physical laws (gravity, slope, permeability). Physics equations are more accurate and explainable for this than a trained ML model, especially with limited training data |
| Input | Predicted rainfall grid from Model 1, DEM flow direction grid, flow accumulation grid, composite runoff coefficient raster |
| Output | Volume of runoff (litres per second) arriving at each drain inlet node for each 15-minute time step |
| Key equation | Q_inlet = C × I × A where A is the upstream contributing area to that inlet, derived from flow accumulation raster |

### 4.4.3 Model 3 — Drain Graph Hydraulic Model (Physics-Based)

| Attribute | Detail |
| --- | --- |
| Purpose | Propagate runoff through the drainage network, identify where pipes exceed capacity and overflow occurs |
| Algorithm | Directed graph traversal with Manning's equation for hydraulic capacity on each edge |
| Data structure | Directed weighted graph: nodes = manholes and inlets, edges = pipes and canals with capacity weights |
| Input | Drain graph structure with edge capacities, runoff volumes from Model 2 at each inlet node |
| Processing | For each node in topological order (upstream to downstream): sum incoming flow from all upstream pipes, calculate current pipe utilization ratio, if total inflow exceeds effective edge capacity then flag overflow with excess volume, propagate remaining volume downstream |
| Output | List of overflow nodes with excess water volume in litres per second, current capacity utilization percentage for every pipe segment |
| Library | NetworkX for graph construction and traversal in Python |

### 4.4.4 Model 4 — Flood Depth and Risk Prediction (XGBoost + Random Forest)

This is the ML layer that converts hydraulic overflow signals into actual flood depth predictions and risk classifications.

Flood Depth Regressor — XGBoost

| Attribute | Detail |
| --- | --- |
| Purpose | Predict actual water accumulation depth in centimetres at each overflow point |
| Algorithm | XGBoost Gradient Boosted Decision Trees (regression) |
| Why XGBoost | Tabular features, continuous output, handles mixed feature types well, fast inference, built-in feature importance for explainability |
| Input features | Excess overflow volume at node, terrain slope at location, nearest road surface runoff coefficient, drain effective capacity utilization, distance to nearest nala or canal, tree canopy cover percentage, historical maximum flood depth at this location from past events, hours since last rainfall event, total rainfall in past 6 hours |
| Output | Predicted water depth in centimetres |
| Training data | Historical flood events with known water depths: 2015 Chennai floods, field survey records, citizen reports with location tags, insurance loss assessment records |

Risk Classifier — Random Forest

| Attribute | Detail |
| --- | --- |
| Purpose | Classify each road segment into risk category for dashboard colour coding and alert triggering |
| Algorithm | Random Forest Classifier |
| Why Random Forest | Robust to noisy features, provides probability estimates not just class labels, good with tabular data, resistant to overfitting |
| Input features | Predicted water depth from XGBoost, road type (arterial/residential/highway), traffic volume estimate, presence of critical infrastructure (hospitals, schools), historical flood frequency at location |
| Output classes | No Risk / Low (0-10cm) / Medium (10-30cm) / High (30-60cm) / Critical (>60cm) |
| Output | Risk class label and probability score 0.0 to 1.0 for each class |

## 4.5 Layer 4 — Output Services

### 4.5.1 GIS Dashboard

▸  Technology: Leaflet.js for interactive web map, React for UI framework, GeoJSON for real-time data serving

▸  Base map: OpenStreetMap tiles

▸  Flood layer: Street segments colour-coded by risk class, refreshed every 5 minutes

▸  Time slider: User can view predictions for any time step in the 0-3 hour window

▸  Info panel: Click any street segment to see predicted depth, risk probability, drain utilization, key contributing factors

▸  Rainfall overlay: Animated radar rainfall heatmap showing where rain is falling

▸  Historical comparison: Toggle to view 2015 actual flood extent alongside current prediction

▸  Filters: Filter by district, risk level, road type

▸  Admin view: Higher resolution data and drain-level details for CMWSSB engineers

### 4.5.2 Alert System

▸  Trigger logic: When any road segment probability crosses configurable thresholds (default 60% for High, 80% for Critical), alerts are generated

▸  Alert channels: SMS via Twilio API, Email via SMTP, Web push notifications via Firebase Cloud Messaging, Dashboard notification panel

▸  Alert content: Location description, predicted depth, time to onset, confidence score, suggested action

▸  Recipient groups: CMWSSB engineers (drain-level alerts), Traffic police (road closure alerts), NDRF teams (critical zone alerts), Citizen subscribers (area-level alerts)

▸  Deduplication: No repeat alert for same location within 30-minute window unless risk level escalates

### 4.5.3 Route Safety API

A REST API endpoint that navigation applications can call to get flood-safe route recommendations.

| Attribute | Detail |
| --- | --- |
| Endpoint | GET /api/v1/safe-route |
| Parameters | from (lat,lon), to (lat,lon), time (ISO timestamp or 'now'), horizon (minutes, default 60) |
| Algorithm | Dijkstra shortest path on OSM road network graph with edge weights modified by flood risk score |
| Edge weight formula | Base travel time + (Risk score × Penalty factor). High-risk segments get very high penalty, effectively blocking them |
| Response | JSON with recommended route as coordinate array, list of avoided segments with flood reason, risk level for recommended route, validity window in minutes |
| Framework | FastAPI (Python) |
| Use cases | Emergency vehicle routing, citizen navigation apps, NDRF deployment planning, public transit rerouting |

## 4.6 Layer 5 — Feedback and Continuous Learning

▸  After each flood event, actual inundation data is collected from Sentinel-1 SAR imagery, citizen reports, and CMWSSB records

▸  Predicted vs actual comparison generates model accuracy metrics (Precision, Recall, F1, RMSE for depth)

▸  New event data is added to training dataset

▸  XGBoost and Random Forest models are retrained on enriched dataset monthly or after each significant event

▸  ConvLSTM is retrained quarterly due to higher computational cost

▸  Model performance is tracked over time on the admin dashboard to demonstrate continuous improvement

# 5. Data Requirements

## 5.1 Complete Dataset List

| Dataset | Source | Format | Free? | Priority |
| --- | --- | --- | --- | --- |
| IMD historical hourly rainfall (5 yrs) | IMD Data Portal / MOSDAC | NetCDF, CSV | Partial | Critical |
| Doppler radar data (2015 event) | IMD Chennai / MOSDAC | HDF5, NetCDF | On request | Critical |
| Cartosat DEM (1-30m) | ISRO Bhuvan Portal | GeoTIFF | Yes | Critical |
| SRTM 30m DEM (fallback) | USGS EarthExplorer | GeoTIFF | Yes | Critical |
| OSM drainage network | OpenStreetMap / Overpass API | GeoJSON, Shapefile | Yes | Critical |
| OSM road network | OpenStreetMap / Overpass API | GeoJSON, Shapefile | Yes | Critical |
| LULC map (Chennai) | Bhuvan / ESA Copernicus | GeoTIFF, Shapefile | Yes | High |
| Sentinel-1 SAR (2015 flood extent) | Copernicus Open Access Hub | GeoTIFF | Yes | High |
| Sentinel-2 NDVI (tree canopy) | Copernicus Open Access Hub | GeoTIFF | Yes | High |
| CMWSSB drain network data | CMWSSB / Municipal records | CAD, PDF, Shapefile | On request | High |
| NDMA flood reports (2015) | NDMA website | PDF | Yes | Medium |
| News archive 2015 floods (location tags) | Web scraping / Wayback Machine | Text, Images | Yes | Medium |
| Rain gauge station data (Chennai) | IMD Chennai regional office | CSV | On request | Medium |
| Road surface type data | OSM surface tags | GeoJSON attribute | Yes | Medium |

## 5.2 Data for Model Training vs Validation

| Split | Data Used | Purpose |
| --- | --- | --- |
| Training (70%) | 2010-2014 rainfall events, minor flood events | Train ML models |
| Validation (15%) | 2015 flood event (first half) | Tune hyperparameters |
| Test (15%) | 2015 flood event (second half), 2021 event | Final model evaluation |

# 6. Machine Learning Models — Complete Summary

| Model | Algorithm | Type | Input | Output | Library |
| --- | --- | --- | --- | --- | --- |
| Rainfall Nowcasting | ConvLSTM | Deep Learning | 12 radar frames (past 3 hrs) | 12 predicted frames (next 3 hrs) | TensorFlow/Keras |
| Surface Runoff Routing | Rational Method + D8 | Physics-Based | Rainfall grid, DEM, Runoff coefficients | Runoff volume at each drain inlet | Rasterio, NumPy |
| Drain Graph Hydraulics | Graph traversal + Manning's | Physics-Based | Drain graph, Inlet runoff volumes | Overflow nodes, Pipe utilization % | NetworkX |
| Flood Depth Prediction | XGBoost (Regression) | ML — Gradient Boosting | 10 tabular features per node | Water depth in cm | XGBoost |
| Risk Classification | Random Forest (Classifier) | ML — Ensemble | Depth + contextual features | Risk class + probability | Scikit-learn |
| Route Finding | Dijkstra / A* | Graph Algorithm | Road network + risk scores | Safest route sequence | NetworkX / OSRM |

## 6.1 Model Input Features Detail — XGBoost Flood Depth Regressor

| Feature | Description | Data Source |
| --- | --- | --- |
| Excess overflow volume | Litres/sec beyond pipe capacity at node | Drain graph model output |
| Terrain slope | Degrees, from DEM | DEM processing |
| Road runoff coefficient | Composite C value (0.0 to 1.0) | OSM + LULC |
| Drain utilization ratio | Current flow / effective capacity (%) | Drain graph model |
| Distance to nala/canal | Metres to nearest major open drain | OSM |
| Tree canopy cover | % canopy within 50m radius | Sentinel-2 NDVI |
| Historical max depth | Maximum recorded depth at this location | Historical flood records |
| Hours since last rain | Dry period before current event | Rain gauge data |
| Total rainfall last 6 hrs | Cumulative mm in past 6 hours | Radar / gauge data |
| Drain age | Years since last CMWSSB record of construction | CMWSSB data / OSM |

## 6.2 Model Evaluation Metrics

| Model | Metric | Target |
| --- | --- | --- |
| ConvLSTM Nowcasting | Critical Success Index (CSI) | > 0.5 for heavy rainfall cells |
| XGBoost Depth | Root Mean Square Error (RMSE) | < 8 cm average depth error |
| XGBoost Depth | Mean Absolute Error (MAE) | < 5 cm |
| Random Forest | F1 Score (weighted) | > 0.75 across all classes |
| Random Forest | Precision for Critical class | > 0.80 (minimize false negatives) |
| Overall System | Flood location detection rate | > 75% of actual flood points predicted |
| Overall System | False alarm rate | < 25% of predicted points should not flood |

# 7. Technology Stack

| Category | Technology | Purpose |
| --- | --- | --- |
| Primary Language | Python 3.10+ | All data processing, ML, and backend |
| ML — Deep Learning | TensorFlow 2.x / Keras | ConvLSTM rainfall nowcasting model |
| ML — Gradient Boosting | XGBoost 1.7+ | Flood depth prediction regression |
| ML — Ensemble | Scikit-learn | Random Forest risk classifier |
| Graph Processing | NetworkX | Drain graph construction and traversal |
| Spatial Data | GeoPandas | Shapefile and GeoJSON processing |
| Raster Processing | Rasterio, GDAL | DEM, radar, and satellite raster handling |
| API Framework | FastAPI | Route safety API and data serving endpoints |
| Database — Spatial | PostgreSQL + PostGIS | Storing all spatial features and predictions |
| Database — Cache | Redis | Caching real-time prediction results |
| Frontend Map | Leaflet.js | Interactive GIS web dashboard |
| Frontend Framework | React.js | Dashboard UI components |
| Scheduling | Apache Airflow / Cron | Orchestrating data pipeline refreshes |
| Alerts — SMS | Twilio API | SMS notifications to authorities |
| Alerts — Push | Firebase Cloud Messaging | Mobile push notifications |
| Containerization | Docker + Docker Compose | Reproducible deployment |
| Version Control | Git + GitHub | Code management |

# 8. Key Differentiators

Most existing flood prediction systems use rainfall + terrain elevation only. This system goes significantly further in three dimensions:

## 8.1 Road Surface Permeability Modelling

The PS itself states that urban flooding is dictated by 'concrete imperviousness.' This system is one of the few to operationalize this by assigning differentiated runoff coefficients to road surface types — concrete, asphalt, paver blocks, and gravel — rather than treating all urban surfaces identically. In Chennai, where road surface quality varies dramatically between arterial roads and residential lanes, this improves runoff estimation accuracy significantly.

## 8.2 Drain Infrastructure Condition Modelling

The PS describes drainage networks as 'heavily strained.' Rather than assuming drains operate at design capacity, this system assigns an effective capacity that accounts for pipe age, material, condition, and blockage probability. This is critical for Chennai, where many drainage structures date from the British era and operate well below their original design specifications.

## 8.3 Tree Canopy Interception

Tree cover intercepts 15 to 40% of rainfall before it reaches the ground. Areas with mature tree canopy generate significantly less runoff than bare concrete areas receiving identical rainfall. By incorporating Sentinel-2 NDVI-derived canopy data, the system captures this effect in the runoff coefficient computation, producing more accurate street-level predictions.

## 8.4 Explainability

Unlike black-box deep learning systems, this solution is explainable at every step. For any predicted flood point, the system can state exactly which drain segment is overflowing, at what utilization percentage, what volume of runoff is arriving from the upstream catchment, and which surface characteristics are contributing most to the runoff volume. This is essential for trust and adoption by municipal engineers.

# 9. SIH Prototype Scope

## 9.1 Study Area

▸  City: Chennai, Tamil Nadu

▸  Study zone: Adyar watershed + T. Nagar + Velachery zone (approximately 10 × 10 km)

▸  This area was severely affected in 2015 and has good OSM drainage coverage

▸  Contains a mix of residential, commercial, and arterial roads for diverse validation

## 9.2 What Will Be Demonstrated

Event Replay Demonstration

Since live radar data may not be accessible during the hackathon, the prototype will use Event Replay — feeding historical 2015 Chennai flood hourly rainfall data into the pipeline as if it were arriving in real time. The system will predict flood locations hour by hour as the event unfolds, and judges can compare predictions against the documented 2015 actual flood locations.

▸  Working drain graph for the 10 × 10 km study area built from OSM data

▸  ConvLSTM nowcasting OR direct historical rainfall replay as input

▸  Surface runoff calculation with road surface and tree cover factors

▸  Drain graph hydraulic simulation identifying overflow nodes

▸  XGBoost depth prediction at overflow points

▸  Live GIS dashboard with street-level risk colour coding

▸  Alert system simulation showing triggered notifications

▸  Route API endpoint returning flood-safe route between two test coordinates

▸  Prediction vs actual 2015 flood extent comparison panel

## 9.3 What is Deferred to Production

▸  Live radar data integration — replaced by historical replay in prototype

▸  Full city drainage network — prototype uses 10 × 10 km study area only

▸  Production-grade alert infrastructure — simulated in prototype

▸  Mobile application — Route API is built but frontend app is deferred

# 10. Drainage Blockage Modelling

Drainage blockage is one of the most impactful and most commonly ignored factors in urban flood prediction. In Chennai specifically, blocked drains are a primary cause of street flooding — not just overcapacity. This system explicitly models blockage as a first-class parameter.

## 10.1 Types of Blockages Modelled

| Blockage Type | Cause | Effect on Model | Data Source |
| --- | --- | --- | --- |
| Solid waste blockage | Garbage, plastic, leaves at inlet grates | Reduces inlet efficiency to 0.0–0.4, surface runoff cannot enter drain | GCC complaint records, solid waste density maps |
| Sedimentation | Silt and sand accumulation inside pipes over years | Reduces internal pipe diameter, cuts hydraulic capacity by up to 50% | CMWSSB maintenance records, pipe age data |
| Root intrusion | Tree roots entering brick or earthen drain joints | Creates partial blockage, increases roughness coefficient | Drain material + tree proximity data |
| Construction debris | Temporary blockage from road or building work | Complete or near-complete inlet blockage | Construction permit records, OSM building data |
| Grease and sludge | Commercial kitchen and sewage discharge in drains | Gradual capacity reduction, localised blockage points | Land use classification (commercial zones) |

## 10.2 Inlet Efficiency Factor

Each drain inlet node is assigned an inlet efficiency factor (IEF) ranging from 0.0 to 1.0. This factor directly scales how much of the arriving surface runoff actually enters the drain:

Actual inlet flow = Surface runoff × Inlet Efficiency Factor (IEF)

When IEF is 0.0, the inlet is fully blocked and all runoff stays on the surface and accumulates. When IEF is 1.0, the inlet is clean and all runoff enters the drain. IEF is computed per node from:

▸  Last cleaning date from GCC records — drains cleaned within 30 days get IEF boost

▸  Historical blockage complaint frequency from Chennai 311-style civic app data

▸  Street garbage density score from solid waste management records

▸  Proximity to construction zones — within 200m gets IEF penalty

▸  Drain material — open concrete drains block less easily than closed pipe drains

## 10.3 Pipe Sedimentation Factor

Inside pipes, sedimentation reduces the effective flow area. Since flow through a pipe scales with the cross-sectional area raised to the power of 5/3 (from Manning's equation), even a modest reduction in diameter causes a large drop in capacity:

| Pipe Age | Estimated Sedimentation | Capacity Remaining |
| --- | --- | --- |
| 0–5 years | 5–10% | ~90–95% of design capacity |
| 5–15 years | 15–25% | ~75–85% of design capacity |
| 15–30 years | 30–45% | ~55–70% of design capacity |
| 30+ years (brick/earthen) | 50–70% | ~30–50% of design capacity |

## 10.4 Interactive Blockage Simulation (Demo Feature)

The GIS dashboard includes an interactive blockage simulation tool. A judge or operator can click any manhole or inlet node on the map and mark it as blocked. The system instantly recalculates the flood prediction for the surrounding area and shows how the flood extent expands when that node is removed from the network. This demonstrates the model's logic visually in under 10 seconds and is a key differentiator in the live demo.

## 10.5 Drain Health Report Card

The system automatically generates a ranked Drain Health Report that lists the top vulnerable drain segments by a combined vulnerability score:

Vulnerability Score = (1 - IEF) × (1 - Capacity ratio) × Upstream catchment area

This report is immediately actionable for CMWSSB maintenance engineers — it tells them exactly which drains to clean or repair first to get the maximum reduction in flood risk across the city.

# 11. 5-Day SIH Execution Plan

The prototype is designed to be built and demonstrated within 5 days at the hackathon venue. The strategy is to use event replay of the 2015 Chennai floods as a stand-in for live radar data, and to scope the study area tightly to the Adyar and Velachery zone (10 × 10 km) where data coverage and historical flood documentation are strongest.

## Day 1 — Data Collection and Environment Setup

Goal: All data downloaded, database running, environment ready

| Task | Tool / Source | Owner | Expected Time |
| --- | --- | --- | --- |
| Download SRTM 30m DEM for Chennai | USGS EarthExplorer | Data Engineer | 1 hour |
| Pull OSM road + drainage network via Overpass API | overpass-api.de | GIS Developer | 1 hour |
| Get 2015 hourly rainfall data | IMD / research paper supplementary data | Data Engineer | 2–3 hours |
| Download Sentinel-1 SAR flood extent (Nov 2015) | Copernicus Open Access Hub | Data Engineer | 1 hour |
| Download Sentinel-2 NDVI for canopy layer | Copernicus Open Access Hub | Data Engineer | 1 hour |
| Download LULC map for study area | Bhuvan / ESA Copernicus | GIS Developer | 1 hour |
| Set up PostgreSQL + PostGIS database | Docker | Backend Developer | 2 hours |
| Set up Python environment and all libraries | pip / conda | All | 1 hour |
| Load all downloaded data into PostGIS | GDAL / GeoPandas | Data Engineer | 2 hours |

## Day 2 — Core Physics Models

Goal: Drain graph built, surface runoff working, blockage factors assigned

| Task | Tool / Library | Owner | Expected Time |
| --- | --- | --- | --- |
| Preprocess DEM — pit fill, flow direction, flow accumulation | Rasterio, pysheds | GIS Developer | 3 hours |
| Build drain graph from OSM network | NetworkX, GeoPandas | GIS Developer | 3 hours |
| Assign pipe capacities using Manning's equation | Python + NumPy | ML Engineer 1 | 2 hours |
| Assign runoff coefficients from LULC and road surface tags | GeoPandas | ML Engineer 2 | 2 hours |
| Compute tree canopy interception from NDVI | Rasterio | ML Engineer 2 | 1 hour |
| Assign inlet efficiency factors (blockage model) | Python + PostGIS | ML Engineer 1 | 2 hours |
| Assign pipe sedimentation factors from OSM age data | Python | ML Engineer 1 | 1 hour |
| Implement Rational Method surface runoff calculator | NumPy | ML Engineer 2 | 2 hours |

## Day 3 — ML Models

Goal: XGBoost and Random Forest trained, drain graph hydraulic simulation running

| Task | Tool / Library | Owner | Expected Time |
| --- | --- | --- | --- |
| Implement drain graph hydraulic simulation (flow routing + overflow detection) | NetworkX + Manning's | GIS Developer | 4 hours |
| Prepare training dataset from 2015 flood records and research papers | Pandas | ML Engineer 1 | 2 hours |
| Engineer all input features for XGBoost model | Pandas + GeoPandas | ML Engineer 2 | 2 hours |
| Train and tune XGBoost flood depth regressor | XGBoost, Scikit-learn | ML Engineer 1 | 2 hours |
| Train and tune Random Forest risk classifier | Scikit-learn | ML Engineer 2 | 1 hour |
| Implement Dijkstra route finding on OSM road network | NetworkX | Backend Developer | 2 hours |
| End-to-end pipeline test run on one historical hour of data | Python | All | 2 hours |

Note: ConvLSTM rainfall nowcasting is not built from scratch. The 2015 hourly rainfall data is fed directly into the pipeline as a replay. This saves 1-2 days of model training time with zero loss to demo quality.

## Day 4 — Outputs and Integration

Goal: Dashboard live, alerts working, Route API responding, full pipeline integrated

| Task | Tool / Library | Owner | Expected Time |
| --- | --- | --- | --- |
| Build Leaflet.js GIS dashboard with flood risk colour overlay | Leaflet.js, React | Frontend Developer | 4 hours |
| Add real-time risk layer refresh (every 5 min simulation) | JavaScript, FastAPI | Frontend Developer | 2 hours |
| Add time slider for 0-3 hour prediction window | Leaflet.js | Frontend Developer | 1 hour |
| Add interactive blockage simulation button on map | Leaflet.js + FastAPI | Frontend + Backend | 2 hours |
| Build FastAPI Route Safety API endpoint | FastAPI, NetworkX | Backend Developer | 2 hours |
| Build alert trigger logic and SMS demo via Twilio | Twilio API, Python | Backend Developer | 2 hours |
| Add economic impact overlay (businesses + estimated loss) | GeoJSON, Leaflet.js | Frontend Developer | 1 hour |
| Integrate full pipeline end to end and test | All | All | 2 hours |

## Day 5 — Validation, Demo Polish, and Presentation

Goal: Accuracy numbers computed, demo rehearsed, presentation ready

| Task | Tool / Library | Owner | Expected Time |
| --- | --- | --- | --- |
| Run full 2015 event replay through pipeline | All | ML Engineers | 2 hours |
| Compare predicted flood locations vs Sentinel-1 actual flood extent | GeoPandas, Rasterio | GIS Developer | 2 hours |
| Compute accuracy metrics (Precision, Recall, F1, RMSE) | Scikit-learn | ML Engineer 1 | 1 hour |
| Generate Drain Health Report Card for top 10 vulnerable segments | Pandas | ML Engineer 2 | 1 hour |
| Record demo video of event replay on dashboard | Screen capture | Frontend Developer | 1 hour |
| Prepare and rehearse live demo flow for jury | All | All | 2 hours |
| Prepare pitch opening — Chennai 2015 footage + system demo hook | All | Team Lead | 1 hour |
| Final bug fixes and dashboard polish | All | All | 2 hours |

## 11.1 What is Built vs Deferred

| Component | Status | Reason |
| --- | --- | --- |
| Drain graph hydraulic model | Built — Day 2-3 | Core of the PS, non-negotiable |
| Surface runoff with blockage model | Built — Day 2 | Key differentiator, fast to build |
| XGBoost depth + Random Forest classifier | Built — Day 3 | Trains in minutes, high impact |
| GIS dashboard with blockage simulation | Built — Day 4 | Visual impact for judges |
| Route Safety API | Built — Day 4 | PS requirement, 50 lines of code |
| Alert system with live SMS demo | Built — Day 4 | High wow factor, Twilio free tier |
| ConvLSTM nowcasting model | Deferred — use replay | Would take 2+ days to train properly |
| Full city drainage network | Deferred — 10x10km only | Scope control, quality over coverage |
| Mobile application | Deferred | API is built, app frontend is deferred |
| Production infrastructure | Deferred | Docker compose is enough for demo |

# 12. Going Beyond Jury Expectations

The following six features are specifically designed to make the jury stop, take notice, and remember this solution. Each one is buildable within the 5-day plan and addresses a dimension that standard hackathon submissions miss entirely.

## 12.1 Live Interactive Blockage Simulation

On the GIS dashboard, any manhole or inlet node on the map is clickable. When a judge clicks a node during the live demo and marks it as blocked, the system recalculates the flood extent for that area within seconds and the map updates to show the expanded flood zone. This demonstrates the model's internal logic more powerfully than any slide or explanation can. No other team will have this level of interactivity.

## 12.2 Economic Impact Overlay

Most teams show where flooding happens. This system also shows what it costs. The dashboard includes an economic impact panel that calculates the number of businesses, residences, schools, and hospitals within the predicted flood zone, along with a rough economic loss estimate based on floor area and sector type. A statement like 'this event affects 2,400 businesses and causes an estimated 6 crore rupees in economic damage' makes the problem tangible for non-technical jury members in a way that a flood map alone cannot.

## 12.3 Drain Health Report Card

The system auto-generates a ranked table of the top 10 most vulnerable drain segments across the study area, scored by a combination of blockage probability, remaining capacity, and upstream catchment area. This is immediately actionable for CMWSSB engineers — it tells them exactly which drains to clean or repair tomorrow morning to get the maximum reduction in city-wide flood risk. This shifts the system from a passive prediction tool to an active planning tool, which is a fundamentally different value proposition.

## 12.4 Explainability Panel

When the system predicts flooding at any location, clicking that point opens an explainability panel showing exactly why: which drain segment is at what percentage of capacity, what the inlet efficiency is at the nearest node, what the upstream runoff coefficient is, and how much the tree canopy is reducing the effective rainfall. This directly satisfies the PS requirement for Explainable AI and builds trust with municipal engineers who need to understand and justify predictions to their superiors.

## 12.5 Infrastructure Recommendation Engine

After running a flood prediction, the system generates a What-If panel showing the impact of targeted infrastructure interventions. For example: 'Cleaning drain segment AB restores full capacity and reduces Velachery flood extent by 34%' or 'Adding a retention pond at coordinates X reduces predicted depth at this intersection from 45cm to 12cm.' This converts the platform from a monitoring tool into a planning tool — a distinction that impresses urban planning and policy judges significantly.

## 12.6 Live SMS Alert During Demo

During the presentation, the team triggers a flood alert from the system using the Twilio SMS integration. An SMS notification arrives on a phone placed on the demo table in real time, visible to all jury members. This 30-second moment makes the entire prototype feel like a deployed, production-ready system rather than a hackathon project. Twilio's free tier fully supports this with no cost.

## 12.7 The Opening That Wins

The presentation opens with 30 seconds of real footage from the 2015 Chennai floods — people on rooftops, cars submerged, hospitals unreachable. Then the presenter says: 'Our system would have predicted this 2 hours and 40 minutes before it happened. Here is the proof.' The 2015 event replay is then run live on the dashboard. This emotional opening — connecting real human suffering to the technical solution — is what separates a winning presentation from a technically correct but forgettable one.

# 13. Suggested Team Roles — 5-Day Hackathon

| Role | Responsibilities |
| --- | --- |
| Data Engineer | Data pipeline, PostGIS setup, DEM and radar data processing |
| ML Engineer 1 | ConvLSTM nowcasting model, model training and evaluation |
| ML Engineer 2 | XGBoost, Random Forest, feature engineering, model integration |
| GIS Developer | Drain graph construction, spatial analysis, Leaflet.js dashboard |
| Backend Developer | FastAPI Route API, alert system, database integration |
| Frontend Developer | React dashboard UI, real-time map updates, demo preparation |

# 14. Conclusion

The Urban Flood Nowcasting System proposed here is a comprehensive, technically grounded solution that directly addresses every requirement of Problem Statement 26085. It moves beyond simple rainfall forecasting to deliver a coupled physics and ML pipeline that predicts street-level inundation 0 to 3 hours in advance.

By incorporating road surface permeability, drainage infrastructure condition, tree canopy interception, and drainage blockage modelling — factors ignored by conventional flood models — the system delivers significantly more accurate, more realistic predictions tailored to the specific conditions of Indian urban drainage infrastructure.

The prototype is scoped for Chennai's Adyar and Velachery zone using the 2015 flood event as training and validation data. The 5-day execution plan is realistic and prioritises the highest-impact components first. The interactive blockage simulation, economic impact overlay, drain health report card, and live SMS alert are designed specifically to exceed jury expectations and demonstrate production-ready thinking.

This platform enables municipal authorities to shift from reactive flood response to proactive, predictive urban flood management — saving lives, reducing economic loss, and accelerating India's ability to manage climate-driven urban flooding at scale.

# Appendix A — Key Terminology

| Term | Definition |
| --- | --- |
| DEM | Digital Elevation Model — a raster grid representing ground surface elevation at each cell |
| ConvLSTM | Convolutional Long Short-Term Memory — a neural network that processes sequences of spatial grids |
| Manning's Equation | A hydraulic formula that calculates flow velocity in open channels and pipes based on slope, roughness, and cross-section |
| Runoff Coefficient | A dimensionless number (0 to 1) representing the fraction of rainfall that becomes surface runoff |
| D8 Algorithm | A terrain analysis algorithm that assigns each grid cell a flow direction toward one of its 8 neighbours based on steepest descent |
| Hydraulic Capacity | The maximum volume of water a pipe or channel can carry per unit time before overflowing |
| Backflow | Reverse flow of water in a pipe when downstream pressure exceeds upstream pressure |
| SAR | Synthetic Aperture Radar — a satellite imaging technique that can detect surface water through cloud cover |
| NDVI | Normalized Difference Vegetation Index — a satellite-derived measure of vegetation density and canopy cover |
| PostGIS | A spatial extension for PostgreSQL that adds support for geographic objects and spatial queries |
| Nowcasting | Very short-range weather prediction (0 to 6 hours ahead) using current observations rather than numerical models |
| Surcharge | Condition when a pipe is flowing completely full under pressure, typically leading to manhole overflow |
| Inlet Efficiency Factor (IEF) | A value from 0.0 to 1.0 representing what fraction of arriving surface runoff actually enters a drain inlet, accounting for blockage |
| Sedimentation Factor | A derating multiplier applied to pipe capacity to account for silt and debris accumulated inside pipes over time |
| Blockage Probability | A computed score per drain node representing the likelihood that the inlet is partially or fully blocked based on maintenance history and local conditions |
| Event Replay | A validation technique where historical rainfall data is fed into a prediction system in sequence to simulate real-time operation and compare predictions against known outcomes |

# Appendix B — Key Data Sources and URLs

| Dataset | URL / Access Method |
| --- | --- |
| ISRO Bhuvan (Cartosat DEM, LULC) | bhuvan.nrsc.gov.in |
| USGS EarthExplorer (SRTM DEM) | earthexplorer.usgs.gov |
| Copernicus Open Access Hub (Sentinel) | scihub.copernicus.eu |
| OpenStreetMap Overpass API | overpass-api.de |
| MOSDAC (IMD radar and satellite data) | mosdac.gov.in |
| IMD Data Portal | imdpune.gov.in |
| NDMA Flood Reports | ndma.gov.in |
| Google Earth Engine (NDVI computation) | earthengine.google.com |

— End of Document —